OPERATION :

These procedures are changed from those specified in the original Heathkit Assembly  Manual.

The meter scales should be interpreted as follows:

METER SWITCH		MEASURES		SCALE READING

POSITION

---------------------------------------------------------------------------------------------------------------------------------

GRID				Grid Current		GRID (white area

Represents 0-100 ma)

PLATE		               	Plate Current		0-1000 ma.

REL PWR			Relative power	               0-1000

Output

SWR				Standing wave		1:1 to 3:1

Ratio

HV				High Voltage		1500-3000

OPERATING PROCEDURE :

Make sure the Linear Amplifier is connected to your exciter properly.  Set all switches and controls as follows:

SWITCH OR			SWITCH OR 			COMMENTS

CONTROL			CONTROL

POSITIONJ

---------------------------------------------------------------------------------------------------------------------------------

OFF = ON			OFF				Linear amplifier line cord

Connected to source

LOAD				6

BAND				Between 10 & 15

TUNE				40 segment

METER			              REL PWR

REL PWR SENS		              FULLY			                            Adjust to keep meter inter

CLOCKWISE			              on scale.                                                                                                                                                                      .

Adjust the exciter for full CW output at 50 MHz.  NOTE:  With the Linear Amplifier turned OFF, the exciter output is fed through the Linear Amplifier to the RF OUTPUT jack.  Also, the REL PWR and SWR functions of the Meter may be use to aid in tuning the exciter.

Reduce the drive level of the exciter to a minimum.

Place the Meter switch of the Linear Amplifier in the PLATE position and push the OFF-ON switch to ON.  With no driver output from the exciter, key the exciter, the Linear Amplifier meter should show idling plate current of approximately 90 ma.

Advance the drive level of the exciter for a 200 ma plate current indication on the meter of the Linear Amplifier.

Quickly switch the Linear Amplifier Meter switch to REL PWR.  If necessary, reduce the REL PWR SENS control setting to keep the meter pointer within the scale.

Adjust the TUNE and LOAD controls for a maximum REL PWR meter indication.

Check the GRID and PLATE meter readings. If the grid reading is greater than midscale reduce the drive level of the exciter to bring grid current within the GRID range of the meter. If the plate current reading is higher than 500 ma, reduce loading by turning the LOAD control counterclockwise.

This procedure of tuning the Linear Amplifier should take only a few seconds, after you go through it a few times.

DISCONNECT THE EXCITER FROM THE AMPLIFIER

Connect a Watt Meter between the Transceiver/Driver and the Amplifier using a 100 Watt  Element for 50 MHz.  Key the driver and increase the indicated output to 20-50 watts.  Reverse the elemnet and key the drive to view the reflected power.  Adjust the cores of the Grid “T”  circuit (previously left at either end of the inductors) for minimum reflected power.  The element can be changed to a lower wattage to bring the VSWR to minimum . Two (2) wattmeters would be ideal for checking input/VSWR and the Amplifier output during this adjustment.

The Linear Amplifier is now loaded for operation.

PLEASE NOTE:  THIS AMPLIFIER IS CAPABLE OF PRODUCING 700 WATTS OUTPUT, HOWEVER THE LIFE EXPECTENCY OF THE 572B TUBES WILL BE SHORTENED.  KEEP  THE GRID DRIVE IN THE SPECIFIED WHITE AREA OF THE METER (0-100 ma) AT ALL TIMES BY INSURING THE PROPER RF DRIVE INPUT TO THE LINIER AMPLIFIER.  



This Linier Amplifier has a SOFT KEY circuit added and can be keyed by any  of the current state of the art Transceivers. (See attached diagrams)